Adaptive TCP Congestion Control Decision Engine

Using Mininet and Machine Learning

Final Project Proposal  |  Computer Networks Course  |  6-Week Timeline  |  2-Member Team

# Abstract

This project develops an Adaptive TCP Congestion Control Decision Engine that uses Mininet to simulate different network conditions and evaluate the performance of TCP Reno, Cubic, and BBR. A Machine Learning model is trained on the collected data to recommend the most suitable TCP congestion control algorithm for a given network scenario, enabling intelligent and adaptive TCP selection.

# 1. Problem Statement

TCP uses congestion control algorithms such as Reno, Cubic, and BBR to regulate data transmission. However, no single algorithm performs best under every network condition:

- Reno, Cubic, and BBR use different congestion-control strategies and can exhibit different performance under varying network conditions.

- Their relative performance depends on factors such as bandwidth, delay, and packet loss.

- Therefore, the most suitable TCP variant may vary depending on the current network environment.

Currently, an operating system uses one fixed congestion control algorithm regardless of changing network conditions. This project builds a system that recommends the most suitable TCP algorithm based on measured network characteristics, using Machine Learning trained on experimentally collected data — rather than assuming beforehand which algorithm is best.

# 2. Objective

- Simulate varying network conditions (bandwidth, delay, packet loss) using Mininet.

- Benchmark TCP Reno, Cubic, and BBR under each condition.

- Collect performance metrics into a structured dataset.

- Train a Random Forest classifier to predict the best-performing TCP variant.

- Evaluate whether the ML-based recommendation outperforms any single fixed algorithm.

# 3. Technology Stack

| Category | Tools |
| --- | --- |
| Operating System | Ubuntu Linux (via WSL2 on Windows, or native/VM) |
| Network Emulation | Mininet, Open vSwitch |
| Traffic Shaping | Linux tc / netem |
| Traffic Generation & Measurement | iperf3, ping, ss |
| Automation | Python |
| Data Processing | Pandas |
| Machine Learning | Scikit-learn (Random Forest, Decision Tree) |
| Visualization | Matplotlib |
| Version Control | Git / GitHub |
| Optional Dashboard | Flask, HTML/CSS/JavaScript |

# 4. Running on Windows — Environment Setup

Mininet depends on Linux kernel features (network namespaces, veth pairs, tc/netem) that are not available natively on Windows. The recommended setup is WSL2 with Ubuntu.

## 4.1 Setup Steps

- Install WSL2: open PowerShell as Administrator and run ‘wsl --install’ (installs Ubuntu by default on modern Windows builds).

- Launch Ubuntu from the Start Menu and complete the initial username/password setup.

- Update packages: ‘sudo apt update && sudo apt upgrade -y’.

- Install Mininet: ‘sudo apt install mininet’.

- Install supporting tools: ‘sudo apt install iperf3 iproute2 python3-pip git’.

- Install Python packages: ‘pip3 install pandas scikit-learn matplotlib’.

- Use VS Code with the WSL extension to edit and run project code directly inside the Ubuntu environment.

## 4.2 Critical Week 1 Check — BBR Availability

Some default WSL2 kernels do not ship with the tcp_bbr module compiled in. This must be verified in Week 1, not discovered mid-experiment in Week 3, since it directly affects Member 1's data collection setup:

sysctl net.ipv4.tcp_available_congestion_control

If ‘bbr’ does not appear in the output, try:

sudo modprobe tcp_bbr
sysctl net.ipv4.tcp_available_congestion_control   # re-check

If BBR still cannot be enabled after this, fall back to a native Ubuntu VM (VirtualBox/VMware) or the official Mininet VM image (mininet.org/download), both of which use a full Linux kernel with BBR support by default. Confirm this early so the choice of environment does not block Week 2–3 work.

Do not proceed to the full experiment setup until Reno, Cubic, and BBR are all confirmed as available.

# 5. System Architecture

Python Automation Script
                  |
                  v
     Mininet Network (Host1 -- Switch -- Host2)
                  |
        tc/netem: Bandwidth / Delay / Loss
                  |
   -------------------------------------------
   |                  |                       |
TCP Reno          TCP Cubic               TCP BBR
   |                  |                       |
   -------------------------------------------
                  |
         iperf3 (throughput, retransmits)
         ping / ss (RTT, run in parallel)
                  |
                  v
        Raw CSV Dataset  -->  Aggregated ML Dataset
                  |
                  v
         Random Forest Classifier
                  |
                  v
         Recommended TCP Variant

# 6. Network Topology

Start with a simple linear Mininet topology — sufficient for studying TCP behavior without unnecessary complexity:

H1 (sender) ------- S1 (switch) ------- H2 (receiver)

# 7. Experimental Design

## 7.1 Parameters Varied

| Parameter | Values Tested |
| --- | --- |
| Bandwidth | 1, 5, 10, 20, 50, 100 Mbps (6 values) |
| Delay | 5, 20, 50, 100, 200 ms (5 values) |
| Packet Loss | 0, 1, 2, 3, 5% (5 values) |
| TCP Variant | Reno, Cubic, BBR (3 values, via iperf3 -C flag) |

## 7.2 Experiment Count

Full grid: 6 bandwidths × 5 delays × 5 losses = 150 network conditions. Each condition is tested with 3 TCP variants, resulting in 450 experimental runs. With 3 repetitions per condition-variant combination, the complete dataset requires 1,350 runs.

## At roughly 15–20 seconds per run, the 450-run single-pass grid would take approximately 2–2.5 hours. With 3 repetitions, the full 1,350-run experiment would take approximately 6–7.5 hours, before accounting for setup and restart overhead. Plan the full run as a background/overnight process in Week 3 rather than running it interactively.

## 7.3 Repetitions (noise control)

Network measurements are noisy. Each (condition, TCP variant) combination should be run 3 times, with the median throughput and retransmission count used as the recorded value. This roughly triples total runtime to ~6–7.5 hours of automated execution — still feasible as an overnight run, and it should be planned into the Week 3 schedule explicitly rather than left as an afterthought.

## 7.4 Pilot Run First

Before committing to the full grid, validate the automation script on a small pilot:

3 bandwidths x 3 delays x 2 losses x 3 variants = 54 experiments

Only scale to the full 450-experiment grid once the pilot completes cleanly end-to-end (topology → shaping → traffic → logging).

# 8. What Happens During One Experiment

# For a given network condition, such as Bandwidth = 20 Mbps, Delay = 100 ms, and Packet Loss = 2%:

# Configure the link using tc/netem with the target bandwidth, delay, and loss.

# Measure the baseline RTT before running the TCP experiments. This baseline RTT will be used as an ML input feature.

# For each TCP variant (Reno, Cubic, BBR), run iperf3 using the -C flag to select the congestion control algorithm.

# Collect throughput and retransmissions from iperf3. Optionally collect per-variant RTT using ping for performance analysis.

# Repeat each condition-variant combination 3 times and use the median throughput and retransmission count as the recorded values.

# Compare the median throughput of Reno, Cubic, and BBR for that network condition and label the highest-performing variant as the best-performing TCP variant.

# 9. Dataset Design

Two datasets are maintained for a cleaner methodology:

## 9.1 Dataset A — Raw Benchmark Dataset

One row per (condition, TCP variant) experiment:

bandwidth, delay, loss, tcp_variant, throughput, rtt, retransmissions
20, 100, 2, reno,  15.2, 103, 25
20, 100, 2, cubic, 17.1, 105, 18
20, 100, 2, bbr,   19.0, 102, 11

## 9.2 Dataset B — ML Dataset

Aggregated per network condition, used for model training:

| bandwidth | delay | loss | baseline_rtt | reno_tp | cubic_tp | bbr_tp | best_variant |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20 | 100 | 2 | 103 | 15.2 | 17.1 | 19.0 | BBR |

## 9.3 Defining “Best”

The primary criterion is highest median throughput per condition. This is the simplest, most defensible starting definition. As a documented extension, a weighted performance score (throughput minus a latency and retransmission penalty) can be discussed as future work, but throughput alone should drive the labels used for training.

# 10. Machine Learning Pipeline

Raw CSV -> Data Cleaning -> Feature Selection -> Train/Test Split
        -> Random Forest -> Prediction -> Evaluation

## Input features: bandwidth, delay, packet loss, and baseline RTT. Output: predicted best TCP variant (Reno / Cubic / BBR) — a multiclass classification problem.

## 10.1 Evaluation Strategy

Rather than a random 80/20 split (which can let very similar experiments leak between train and test), hold out entire network conditions from training. Report accuracy, precision, recall, F1 score, confusion matrix, and feature importance on this held-out set, so the claim is genuinely “evaluated on previously unseen network conditions.”

## 10.2 Baseline Comparison (most important evaluation)

ML accuracy alone is not a sufficient result for a Computer Networks project. The key experiment is comparing average throughput achieved by:

- Always Reno

- Always Cubic

- Always BBR

- ML Decision Engine (adaptive selection)

on the held-out test conditions. This demonstrates whether adaptive selection actually improves performance over any fixed algorithm — the central claim of the project.

# 11. Team Division

| Member 1 — Networking & Experimentation | Member 2 — ML & Analysis |
| --- | --- |
| Mininet setup and topology; tc/netem configuration; iperf3 + ping automation (with repetitions); TCP variant configuration; raw dataset output | Dataset cleaning and aggregation; Random Forest / Decision Tree training; held-out evaluation and baseline comparison; graphs; prediction interface |

Both members should understand the full pipeline end-to-end — either member should be able to explain, for example, why BBR behaves differently from Cubic under high latency.

# 12. Six-Week Timeline

| Week | Focus | Target |
| --- | --- | --- |
| 1 | Learn TCP congestion control fundamentals + Mininet/iperf3 basics. Set up WSL2 + Mininet. Verify BBR availability. Build 2-host topology. | First successful iperf3 run; BBR confirmed available |
| 2 | Learn tc/netem. Test bandwidth/delay/loss injection. Run 54-experiment pilot grid. | Automated pilot experiment works end-to-end |
| 3 | Scale to the full 150-condition grid, resulting in 450 TCP runs per repetition and 1,350 total runs across 3 repetitions. Generate raw and aggregated datasets. | Complete experimental dataset |
| 4 | EDA, throughput/delay/loss plots. Train baseline Decision Tree and Random Forest. | First working ML model |
| 5 | Tune model. Held-out evaluation. Run baseline comparison (fixed algorithms vs ML). | Demonstrate whether ML selection provides a real advantage |
| 6 | Build prediction interface. Finalize graphs, report, architecture diagram, and demo. | Final system, report, and presentation ready |

# 13. Scope Boundaries

## 13.1 Minimum Viable Project (must complete)

Mininet topology → network condition emulation → Reno/Cubic/BBR experiments → automated data collection → dataset → Random Forest → TCP recommendation → baseline comparison → performance evaluation.

## 13.2 Stretch Goals (only if ahead of schedule)

- Flask dashboard showing live network conditions and the model's recommendation.

- Automatic adaptive switching: apply the model's recommended TCP variant, re-measure, and repeat as a closed loop.

- Additional TCP variants (Vegas, Westwood) if supported by the kernel.

- Feature importance visualization to explain model decisions.

## 13.3 Explicitly Out of Scope

Deep learning, reinforcement learning, SDN/Ryu controllers, 10+ TCP algorithms, a complex web application, cloud deployment, large multi-router topologies, real hardware testbeds, and automatic kernel-level TCP switching in production. The core project above is sufficient in depth for a 6-week, 2-person course project.

# 14. Why This Project Stands Out

- Combines network emulation with a self-generated dataset rather than a public one.

- Integrates Computer Networks, Linux systems work, Python automation, and Machine Learning end-to-end.

- Goes beyond classification accuracy to prove real-world impact via baseline comparison against fixed algorithms.

- Uses a defensible held-out evaluation methodology rather than a naive random split.

- Has a clear, well-scoped extension path (adaptive closed-loop switching) for future work or a viva discussion point.